Tree CO2 Calculations

Savings over first ten years= 23.2 lbs carbon1 tree44 units carbon dioxide12 unit carbon1 metric ton2204.6 lbs=0.0386 metric tons per tree

CO2 savings per year= 0.0386 metric tons per tree10 years2204.6 lbs1 metric ton=8.51 lbs per tree per year over the first ten years

Note: The initial statistic, 23.2 pounds of carbon per tree, accounts for many trees not surviving the full ten years; it assumes that 68% of trees survive five years, and 59% survive ten years.

Source: http://www.epa.gov/cleanenergy/energy-resources/refs.html#seedlings

Water Bottle CO2 Calculations

CO2 savings = 2.07 metric tons CO2 saved1 short ton PET reduced2204.62 lbs CO21 metric ton CO21 short ton PET2000 lbs PET=2.28 lbs CO2 per lb of PET reduced

Source: http://www.epa.gov/climatechange/wycd/waste/calculators/Warm_Form.html

CFL Bulb Calculations

Sources:

Wattage of bulbs can be found at http://www.duke-energy.com/pdfs/Duke_CFLflyer_406_generic.pdf.

Carbon dioxide emissions due to electricity use can be found at http://oaspub.epa.gov/powpro/ept_pack.charts.

Estimates for lifetime of CFL bulbs found at http://www.energystar.gov/index.cfm?c=cfls.pr_cfls_savings.

Shopping Bag Calculations

| Information | Number | Source or calculation |
| --- | --- | --- |
| Avg. # of paper bags per week, per person | 2 bags | Source 1 |
| Avg. # of plastic bags per week, per person | 7 bags | Source 1 |
| Greenhouse Gas Emissions (CO2 Equiv. Tons) to manufacture 1000 paper shopping bags | 0.08 tons | Source 2 |
| Greenhouse Gas Emissions (CO2 Equiv. Tons) to manufacture 1000 plastic shopping bags | 0.04 tons | Source 2 |
| Greenhouse Gas Emissions (CO2 Equiv. lbs) to manufacture 1 paper shopping bag | 0.16 lbs | 0.08 tons CO21000 paper bags2000 lbs1 ton CO2=0.16 lbs |
| Greenhouse Gas Emissions (CO2 Equiv. lbs) to manufacture 1 plastic shopping bag | 0.08 lbs | 0.04 tons CO21000 paper bags2000 lbs1 ton CO2=0.08 lbs |
| Avg. # of paper bags per year, per person | 104 bags | 2 paper bags1 week52 weeks1 year=104 bags/yr |
| Avg. # of plastic bags per year, per person | 364 bags | 7 plastic bags1 week52 weeks1 year=364 bags/yr |
| Avg. lbs CO2 from manufacture of paper bags used per year, per person | 16.64 lbs | 104 paper bags1 year0.16 lbs1 bag=16.64 lbs |
| Avg. lbs CO2 from manufacture of plastic bags used per year, per person | 29.12 lbs | 364 plastic bags1 year0.08 lbs1 bag=29.12 lbs |
| Net average lbs CO2 from manufacture of paper and plastic bags used per year, per person | 45.76 lbs | 16.64 lbs + 29.12 lbs = 45.76 lbs |

Source 1: http://www.carbonrally.com/challenges/5-reusable-grocery-bags

Source 2: http://www.savetheplasticbag.com/UploadedFiles/2007%20Boustead%20report.pdf, page 4

Alternative Transportation Calculations

Sources:

http://www.epa.gov/cleanenergy/energy-resources/refs.html

http://www.gasbuddy.com/GB_Price_List.aspx

Showerhead Calculations

CO2 savings (gas)= 1440 therms1 AF1 AF325851.427 gal# gallons saved10.005302 metric tons CO21 therm2204.62 lbs CO21 metric ton

CO2 savings (electric) = 42361 kWh1 AF1 AF325851.427 gal# gallons saved10.0006896 metric tons CO21 therm2204.62 lbs CO21 metric ton

Money savings (gas)= 1440 therms1 AF1 AF325851.427 gal# gallons saved1$ price1 therm

Money savings (electric)= 42361 therms1 AF1 AF325851.427 gal# gallons saved1$ price1 kWh

Sources:

http://www.westernresourceadvocates.org/water/CWCBe-wstudy.pdf

http://data.bls.gov/search/query/results?cx=013738036195919377644%3A6ih0hfrgl50&q=average+energy+prices